CODE:

ist p=18f458

#include <p18f458.inc>

;;;;;;DEFINEING LABLES==================================

Lcd_data equ PORTD

Lcd_CTRL equ PORTC

RS EQU RC0

RW EQU RC1

EN EQU RC2

COUNT EQU 0X20

COUNT1 EQU 0X21

ISRC EQU 0X22

;;;;;; ORGINAL LOCATIONS================================

ORG 0X00

GOTO MAIN

ORG 0X08

GOTO ISR

;;;;;; MAIN=============================================

MAIN

;;;;;PORTS INTERFACEING

BANKSEL TRISD

CLRF TRISD

BANKSEL TRISC

CLRF TRISC

BANKSEL TRISB

movlw 0xF0

movwf TRISB

BANKSEL TRISA

CLRF TRISA

;;;;;;;PORTB PULLUP

BCF INTCON2,RBPU

;;;;;;;TIMERON

BANKSEL T1CON

MOVLW   0x31

MOVWF   T1CON

BANKSEL TMR1L

MOVLW 0X00

MOVWF TMR1H

MOVLW 0X00

MOVWF TMR1L

BSF T1CON, TMR1ON

;;;;;;;TIMER INTERUPTS

BSF INTCON, GIE

BSF PIE1, TMR1IE

BSF INTCON, PEIE

;;;;;;;LCD INTERFACING

BCF Lcd_data,EN

CALL LDELAY

MOVLW 0X38

CALL COMWRT

CALL LDELAY

MOVLW 0X0C

CALL COMWRT

CALL DELAY

MOVLW 0x80

CALL COMWRT

CALL DELAY

MOVLW 'E'

CALL DATAWRT

MOVLW 'N'

CALL DATAWRT

CALL DELAY

MOVLW 'T'

CALL DATAWRT

MOVLW 'E'

CALL DATAWRT

MOVLW 'R'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 'P'

CALL DATAWRT

MOVLW 'I'

CALL DATAWRT

MOVLW 'N'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW '*'

CALL DATAWRT

MOVLW '*'

CALL DATAWRT

MOVLW '*'

CALL DATAWRT

MOVLW '*'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 0XC0

CALL COMWRT

;;;;;;;;;;;;;;;;;;;;;;;;;;KEY MAIN LOOP================================

key1

MOVLW 0XFE

movwf PORTB

BTFSS PORTB,0

goto checkcol

bra ROW1

checkcol

BTFSS PORTB,5

goto LCD0

BRA ROW1

;R1C0

ROW1

MOVLW 0XFD

movwf PORTB

BTFSS PORTB,1

goto checkcol2

bra ROW2

checkcol2

BTFSS PORTB,5

goto LCD1

BRA ROW2

;R2C0

ROW2

MOVLW 0XFB

movwf PORTB

BTFSS PORTB,2

goto checkcol3

BRA ROW3

checkcol3

BTFSS PORTB,5

goto LCD2

BRA ROW3

;R3C0

ROW3

MOVLW 0XF7

movwf PORTB

BTFSS PORTB,3

goto checkcol4

checkcol4

BTFSS PORTB,5

goto ENTER

BRA R0C1

;R0C1

R0C1

MOVLW 0XFE

movwf PORTB

BTFSS PORTB,0

goto checkco4

BRA R1C1

checkco4

BTFSS PORTB,6

goto LCD4

BRA R1C1

;R1C1

R1C1

MOVLW 0XFD

movwf PORTB

BTFSS PORTB,1

goto checkcol5

BRA R2C1

checkcol5

BTFSS PORTB,6

goto LCD5

BRA R2C1

;R2C1

R2C1

MOVLW 0XFB

movwf PORTB

BTFSS PORTB,2

goto checkcol6

BRA R3C1

checkcol6

BTFSS PORTB,6

goto LCD6

BRA R3C1

;R3C1

R3C1

MOVLW 0XF7

movwf PORTB

BTFSS PORTB,3

goto checkcol7

BRA R0C2

checkcol7

BTFSS PORTB,6

goto LCD7

BRA R0C2

;R0C2

R0C2

MOVLW 0XFE

movwf PORTB

BTFSS PORTB,0

goto checkco8

BRA R1C2

checkco8

BTFSS PORTB,7

goto LCD8

BRA R1C2

;R1C2

R1C2

MOVLW 0XFD

movwf PORTB

BTFSS PORTB,1

goto checkcol9

BRA R2C2

checkcol9

BTFSS PORTB,7

goto LCD9

BRA R2C2

;R2C2

R2C2

MOVLW 0XFB

movwf PORTB

BTFSS PORTB,2

goto checkcol10

BRA R3C2

checkcol10

BTFSS PORTB,7

goto LCD10

BRA R3C2

;R3C2

R3C2

MOVLW 0XF7

movwf PORTB

BTFSS PORTB,3

goto checkcol11

checkcol11

BTFSS PORTB,7

goto LCD11

BRA key1

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;LCD KEY DISPLAYS================================

;;;;;;LCDS ARE GIVEN FORCED LOCATIONS AS OUR LOGIC IS LIKE A GAME YOU HAVE TO CHOOSE THE RIGHT KEYS FROM 11 KEYS FOR THE GATE TO OPEN ITS LIKE MATCH THE RIGHT KEYS

;;;;;; ALSO LCDS ARE LIKE LOCKS IN OUR CASE! AND SAVED PIN IS 1,5,8,0 THESE PINS ARE GIVEN PORTC,4,5,6,7 LOCATION IF ALL OF THEM ARE SET AND ENTERED IT GIVES ACCESS

;;;;;; WHILE ALL THE OTHER KEYS ARE GIVEN PORTC,3 LOCATIONS IF IT IS SET THE "WRONG PIN TIMEUP" IS DISPLAYED AS OUR CODE RUN ON INFINITY TIME IT ONLY STOPS WHEN EITHER

;;;;;; ACCESS GRANTED,ACCESS DENIED OR WRONG PIN TIMEUP IS ACTIVATED. IF YOU PRESS 1,5 AND PRESS ENTER IT DISPLAYS ACCESS DENIED AS 4 KEYS WERE NOT PRESSED AND 2 OFF THEM

;;;;;; WERE CORRECT, ACCESS DENIED WILL ONLY OCCURE IF YOU ONT FILL THE RIGHT KEYS PERFECTLY

;LCD0

LCD0:

BSF PORTC,5

movlw 0xC0

call COMWRT

MOVLW '1'

CALL DATAWRT

CALL DELAY

return

;LCD1

LCD1:

bsf PORTC,3     ;LINKED TO PORTC,3

movlw 0xC1

call COMWRT

MOVLW '4'

CALL DATAWRT

CALL DELAY

RETURN

;LCD2

LCD2:

bsf PORTC,3     ;LINKED TO PORTC,3

movlw 0xC2

call COMWRT

MOVLW '7'

CALL DATAWRT

CALL DELAY

RETURN

;;;;;;;;;;;;;;;;;;;;;; ENTER KEY;;;;;;;;;;;;;;;;;;;

ENTER

BTFSS PORTC, 4       ; Check if PORTC.4 is set

GOTO ACCESS_DENIED   ; If not, go to "Access Denied"

BTFSS PORTC, 5       ; Check if PORTC.5 is set

GOTO ACCESS_DENIED   ; If not, go to "Access Denied"

BTFSS PORTC, 6       ; Check if PORTC.6 is set

GOTO ACCESS_DENIED   ; If not, go to "Access Denied"

BTFSS PORTC, 7       ; Check if PORTC.7 is set

GOTO ACCESS_DENIED   ; If not, go to "Access Denied"

BTFSC PORTC, 3       ; Check if PORTC.3 is CLEAR

GOTO ACCESS_DENIED   ; If not, go to "Access Denied"

; If all pins are set, access is granted

CALL DISP            ; Call the DISP subroutine to display "Access Granted"

GOTO key1

;;;;;;;;; ACCESS DENIED===========================

ACCESS_DENIED

BSF PORTA,1          ; LED RED ON TO INDICATE ACCESS DENIED

MOVLW 0XC0

CALL COMWRT

CALL DELAY

MOVLW ' '

CALL DATAWRT

MOVLW 'A'

CALL DATAWRT

MOVLW 'C'

CALL DATAWRT

MOVLW 'C'

CALL DATAWRT

CALL DELAY

MOVLW 'E'

CALL DATAWRT

MOVLW 'S'

CALL DATAWRT

MOVLW 'S'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 'D'

CALL DATAWRT

MOVLW 'E'

CALL DATAWRT

MOVLW 'M'

CALL DATAWRT

MOVLW 'I'

CALL DATAWRT

MOVLW 'E'

CALL DATAWRT

MOVLW 'D'

CALL DATAWRT

CALL DELAY

BRA key1

;;;;;;;;; ACCESS GRANTED====================================================

DISP

BSF PORTA,0          ; LED GREEN ON TO INDICATE ACCESS GRANTED

MOVLW 0XC0

CALL COMWRT

CALL DELAY

MOVLW ' '

CALL DATAWRT

MOVLW 'A'

CALL DATAWRT

MOVLW 'C'

CALL DATAWRT

MOVLW 'C'

CALL DATAWRT

CALL DELAY

MOVLW 'E'

CALL DATAWRT

MOVLW 'S'

CALL DATAWRT

MOVLW 'S'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 'G'

CALL DATAWRT

MOVLW 'R'

CALL DATAWRT

MOVLW 'A'

CALL DATAWRT

MOVLW 'N'

CALL DATAWRT

MOVLW 'T'

CALL DATAWRT

MOVLW 'E'

CALL DATAWRT

MOVLW 'D'

CALL DATAWRT

CALL DELAY

BRA key1                     ; BRA TO LOOP

;;;;;;;;;;;;;;;;;;;;;;NOT USED AS * IS OUR ENTER KEY =====================

;LCD3:

;movlw 0xC3

;call COMWRT

;MOVLW '*'

;CALL DATAWRT

;CALL DELAY

;RETURN

;LCD4

LCD4:

bsf PORTC,3;    LINKED TO PORTC,3

movlw 0xC4

call COMWRT

MOVLW '2'

CALL DATAWRT

CALL DELAY

return

;LCD5

LCD5:

BSF PORTC,4

movlw 0xC5

call COMWRT

MOVLW '5'

CALL DATAWRT

CALL DELAY

RETURN

;LCD6

LCD6:

BSF PORTC,6

movlw 0xC6

call COMWRT

MOVLW '8'

CALL DATAWRT

CALL DELAY

RETURN

;LCD7

LCD7:

BSF PORTC,7

movlw 0xC7

call COMWRT

MOVLW '0'

CALL DATAWRT

CALL DELAY

RETURN

;LCD8

LCD8:

BSF PORTC,3;   LINKED TO PORTC,3

movlw 0xC8

call COMWRT

MOVLW '3'

CALL DATAWRT

CALL DELAY

return

;LCD9

LCD9:

bsf PORTC,3;   LINKED TO PORTC,3

movlw 0xC9

call COMWRT

MOVLW '6'

CALL DATAWRT

CALL DELAY

RETURN

;LCD10

LCD10:

bsf PORTC,3;   LINKED TO PORTC,3

movlw 0xCA

call COMWRT

MOVLW '9'

CALL DATAWRT

CALL DELAY

RETURN

;LCD11

LCD11:

bsf PORTC,3;   LINKED TO PORTC,3

movlw 0xCB

call COMWRT

MOVLW '#'

CALL DATAWRT

CALL DELAY

RETURN

;;;;;;;;;;; INTRUPT SERVICE ROUTINE TO DISPLAY WRONG PIN TIME UP

ISR

BTFSS   INTCON, TMR1IF

RETFIE

BCF     INTCON, TMR1IF

BTFSC PORTC,3

GOTO LOOP1

bra key1

LOOP1

GOTO TIMEUP

;;;;;;;;;;;;;;;;;;;;;;;;; DISPLAY WRONG PIN TIMEUP

TIMEUP

MOVLW 0X38

CALL COMWRT

CALL LDELAY

MOVLW 0X0f

CALL COMWRT

CALL DELAY

MOVLW 0xC0

CALL COMWRT

CALL DELAY

MOVLW 'W'

CALL DATAWRT

MOVLW 'R'

CALL DATAWRT

CALL DELAY

MOVLW 'O'

CALL DATAWRT

MOVLW 'N'

CALL DATAWRT

MOVLW 'G'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 'P'

CALL DATAWRT

MOVLW 'I'

CALL DATAWRT

MOVLW 'N'

CALL DATAWRT

MOVLW ' '

CALL DATAWRT

MOVLW 'T'

CALL DATAWRT

MOVLW 'I'

CALL DATAWRT

MOVLW 'M'

CALL DATAWRT

MOVLW 'E'

CALL DATAWRT

MOVLW 'U'

CALL DATAWRT

MOVLW 'P'

CALL DATAWRT

btg PORTA,2 ;  BLUE LED ON TO INCICATE WRONG PIN

bra key1

;;;;;;;;;;;;;; COMMAND WRITE FOR LED====================================

COMWRT

MOVWF Lcd_data

BCF Lcd_CTRL, RS

BCF Lcd_CTRL, RW

BSF Lcd_CTRL, EN

CALL SDELAY

BCF Lcd_CTRL, EN

RETURN

;;;;;;;;;;;;;; DATAWRITE FOR LED============================================

DATAWRT

MOVWF Lcd_data

BSF Lcd_CTRL, RS

BCF Lcd_CTRL, RW

BSF Lcd_CTRL, EN

CALL SDELAY

BCF Lcd_CTRL, EN

RETURN

;;;;;;;;;;;;;; DELAYS=======================================================

LDELAY

MOVLW d'30'

MOVWF COUNT

loop1

DECFSZ  COUNT, f

GOTO    loop1

RETURN

SDELAY:

MOVLW d'20'

MOVWF COUNT

loop2:

DECFSZ  COUNT, f

GOTO    loop2

RETURN

DELAY:

MOVLW d'10'

MOVWF COUNT

loop3:

DECFSZ   COUNT, f

GOTO    loop3

RETURN

;============================      END OF PROGRAM     =======================

END

CORRECT PIN IS 1,5,8,0

If clicked other then = wrong pin timeup will show up

If clicked 2 keys which are right like 1,5 if you enter without filing the rest to pins it will show

= access denied

If 1,5,8,0 is pressed =access granted

ACCESS GRANTED:

WRONG PIN TIMEUP:

ACCESS DENIED